Power Distribution Board (PDB)

Engineering Requirements and Design Guide

Revision G - RP2350 Controller, Per-Rail MOSFET Switching, Buffered GPIO, Buffered I2C

Strict design-control document for schematic, PCB, firmware, bring-up, and mentorship use

| Revision | Major change |
| --- | --- |
| Rev G | Replaced onboard RP2040/Pico concept with bare RP2350A baseline controller. Updated GPIO allocation, I2C topology, Zephyr bindings, KiCad sheet plan, and LCSC part list. Kept six drivetrain MOSFET branches and split ARM/SCIENCE rails. Removed MCP23017 from baseline. |

# Table of Contents

1. How to Use This Document

2. Locked System Requirements

3. System Block Diagram / Architecture

4. Part Selection and LCSC/JLCPCB Procurement Table

5. Sensor Specifications and I2C Address Map

6. RP2350 GPIO Allocation and Software Bindings

7. Connection Requirements and PCB Guidelines

8. KiCad Schematic Sheet Plan

9. Software Implications

10. Validation and Bring-Up Plan

11. Design Checklist / Assignment Rubric

12. Open Verification Items

13. References

# 1. How to Use This Document

Use Sections 2 and 3 to teach the architecture before opening KiCad.

Use Sections 4 through 6 to lock parts, sensors, I2C buses, I2C addresses, and GPIO usage.

Use Sections 7 and 8 as schematic and PCB layout requirements.

Use Sections 9 and 10 as firmware and bring-up requirements.

Use Section 11 as the assignment checklist and review rubric.

Do not order the PCB until Section 12 is resolved or explicitly accepted by the mentor.

# 2. Locked System Requirements

## 2.1 Controller Requirement

The baseline controller shall be a bare Raspberry Pi RP2350A placed directly on the PDB.

The RP2350A shall replace the previous RP2040 baseline.

The baseline shall use the RP2350A QFN-60 package because it has enough GPIO and is currently listed in stock at LCSC.

RP2350B may be used only as an optional future pin-rich variant; it is not the baseline because its LCSC page currently shows not available.

The schematic shall follow the Raspberry Pi RP2350 hardware design guide and the Bronco Space Lab BLVLink hardware v2.0 reference design for RP2350 support circuitry.

The controller shall expose USB-C for USB device/programming/serial connection to the Jetson.

The controller shall support Zephyr/F Prime development. The software baseline shall use RP2350/Raspberry Pi Pico 2 Zephyr support as the closest reference target.

## 2.2 Battery Domains

| Domain | Nominal input | Input connector | Purpose |
| --- | --- | --- | --- |
| Logic/comms | 22.2 V / 6S LiPo | XT60 | Jetson, network switch, 12 V POE antenna, RP2350 housekeeping, aux output |
| Drivetrain | 14.4 V / 4S LiPo | XT60 | Six independently switched drive motor-controller outputs |
| Arm/science | 14.4 V / 4S LiPo | XT60 | Independent ARM and SCIENCE switched outputs |

## 2.3 Logic/Comms Requirements

Jetson, network switch, antenna/POE, and RP2350 housekeeping shall come up automatically when the logic input is present and E-stop is not asserted.

The POE/antenna rail shall be generated from the 22.2 V logic rail and shall output 12 V for a 10 W antenna load.

Logic-side user outputs shall use robust screw terminal blocks unless a specific load requires a different connector.

The aux 22.2 V output shall be RP2350-controlled and E-stop-gated.

The RP2350 shall be powered from onboard housekeeping power, not from Jetson USB.

## 2.4 Drivetrain Requirements

The drivetrain shall use six independent high-side MOSFET branches, one per motor-controller output.

Each drivetrain branch shall use one IPT007N06N high-power N-channel MOSFET or approved equivalent.

Each drivetrain branch shall use one high-side N-MOSFET gate driver.

Each drivetrain branch shall have one dedicated native RP2350 DRIVE_EN_x GPIO.

Each DRIVE_EN_x signal shall pass through the GPIO buffer and E-stop gate before reaching the gate-driver input.

Each drivetrain branch shall include one INA780x I2C current/voltage/power monitor after the MOSFET switch.

Each drivetrain branch output shall use a vertical through-hole XT60 connector facing up.

No TPS1HB08 shall be used for drivetrain motor-controller switching.

## 2.5 Arm/Science Requirements

The arm/science domain shall be split into two independent rails: ARM and SCIENCE.

The ARM rail shall use the same MOSFET + gate driver + INA780x pattern as the drivetrain branches.

The SCIENCE rail shall use the same MOSFET + gate driver + INA780x pattern as the drivetrain branches.

ARM_EN and SCIENCE_EN shall be independent native RP2350 GPIO outputs.

ARM_EN and SCIENCE_EN shall pass through the GPIO buffer and E-stop gate before reaching the gate-driver inputs.

Each rail shall have its own INA780x monitor and high-current output connector.

## 2.6 E-stop Requirements

The PDB shall expose a two-pin screw terminal for the external E-stop switch.

The E-stop switch is a two-prong switch that pulls the E-stop node to GND when pressed.

The E-stop node shall be pulled high during normal operation and pulled low when pressed.

When E-stop is pressed, all functional outputs shall turn off, including compute/comms, to satisfy the stricter rules interpretation.

The E-stop hardware path shall not depend on RP2350 firmware, Jetson software, USB, or I2C.

E_STOP_OK shall be routed to every controlled output enable gate.

Software high-power-disable shall exist, but it shall not replace the hardware E-stop function.

## 2.7 GPIO and Control Requirements

The baseline design shall not use MCP23017 or any other GPIO expander.

All required enable GPIO shall be native RP2350 GPIO pins.

All RP2350-controlled output GPIO shall pass through SN74LVC244APWR non-inverting GPIO buffers before downstream logic.

Buffered enable outputs shall pass through SN74LVC08APWR E-stop gates before reaching gate-driver or load-switch control inputs.

The GPIO buffer and AND gates shall be powered from the 3.3 V housekeeping rail.

Gate-driver logic inputs shall be compatible with the 3.3 V buffered control chain.

Gate-driver supply requirements shall be verified from the selected driver datasheet. Do not assume the gate-driver power pin can use 3.3 V unless the selected driver explicitly supports that and still fully enhances the MOSFET.

# 3. System Block Diagram / Architecture

## 3.1 Top-Level System Tree

PDB
|
|-- 22.2 V LOGIC/COMMS INPUT (6S LiPo, XT60)
|   |-- E-stop-enabled logic bus
|   |-- always-on housekeeping buck -> 3.3 V -> RP2350 + buffers + I2C logic
|   |-- always-on protected/regulator path -> Jetson output
|   |-- always-on protected/regulator path -> network switch output
|   |-- always-on 12 V regulator path -> POE/antenna output, 10 W load
|   `-- RP2350-controlled AUX_22 output
|
|-- 14.4 V DRIVETRAIN INPUT (4S LiPo, XT60)
|   |-- DRIVE branch 1: gate driver -> IPT007N06N -> INA780x -> vertical XT60
|   |-- DRIVE branch 2: gate driver -> IPT007N06N -> INA780x -> vertical XT60
|   |-- DRIVE branch 3: gate driver -> IPT007N06N -> INA780x -> vertical XT60
|   |-- DRIVE branch 4: gate driver -> IPT007N06N -> INA780x -> vertical XT60
|   |-- DRIVE branch 5: gate driver -> IPT007N06N -> INA780x -> vertical XT60
|   `-- DRIVE branch 6: gate driver -> IPT007N06N -> INA780x -> vertical XT60
|
`-- 14.4 V ARM/SCIENCE INPUT (4S LiPo, XT60)
    |-- ARM rail: gate driver -> IPT007N06N -> INA780x -> high-current output
    `-- SCIENCE rail: gate driver -> IPT007N06N -> INA780x -> high-current output

## 3.2 Drivetrain Branch Stack

For each branch x = 1..6:

14.4 V DRIVETRAIN BUS
   |
IPT007N06N high-side N-MOSFET
   |
INA780x I2C power monitor
   |
vertical through-hole XT60 output
   |
motor controller

## 3.3 Drivetrain Control Stack

For each branch x = 1..6:

RP2350 DRIVE_EN_x
   |
SN74LVC244APWR GPIO buffer
   |
SN74LVC08APWR AND gate with E_STOP_OK
   |
high-side N-MOSFET gate driver input
   |
IPT007N06N gate

## 3.4 Arm/Science Stack

14.4 V ARM/SCIENCE INPUT
   |
   |-- ARM_EN path:
   |     RP2350 -> buffer -> E-stop gate -> gate driver -> IPT007 -> INA780x -> ARM output
   |
   `-- SCIENCE_EN path:
         RP2350 -> buffer -> E-stop gate -> gate driver -> IPT007 -> INA780x -> SCIENCE output

## 3.5 RP2350 Controller Stack

22.2 V logic input
   |
housekeeping regulator -> 3.3 V
   |
RP2350A custom controller circuit
   |-- external QSPI flash
   |-- 12 MHz crystal
   |-- USB-C device connector
   |-- SWD header
   |-- BOOTSEL / RUN buttons or pads
   |-- I2C0 -> TCA4307 -> high-current INA780x telemetry
   |-- I2C1 -> TCA4307 -> temperature / logic telemetry
   `-- native GPIO -> SN74LVC244 -> SN74LVC08 -> gate drivers

## 3.6 E-stop Stack

E-stop screw terminal:

3.3 V pull-up / bias network
   |
E_STOP_NODE ---- external 2-prong E-stop switch ---- GND when pressed
   |
E_STOP_OK signal
   |-- to RP2350 E_STOP_STATUS input
   |-- to all SN74LVC08 E-stop gate inputs
   `-- to full-system disconnect logic, relay, or approved competition disconnect circuit

# 4. Part Selection and LCSC/JLCPCB Procurement Table

LCSC/JLCPCB part numbers are included where verified during document generation. Stock shall still be checked at design freeze.

| Function | MPN | LCSC # | Package / Style | Qty | Requirement |
| --- | --- | --- | --- | --- | --- |
| Controller MCU | RP2350A | C42411118 | QFN-60-EP 7x7 | 1 | Baseline controller. Enough GPIO for this architecture. |
| Optional controller variant | RP2350B | C42415655 | QFN-80-EP 10x10 | 0 | Optional future variant with 48 GPIO. Not baseline; LCSC currently shows not available. |
| External QSPI flash | W25Q128JVSIQ | C97521 | SOIC-8-208mil | 1 | Baseline external flash for RP2350A. Final capacity may be changed if Zephyr/F Prime image size requires it. |
| 12 MHz crystal | ABM8-272-T3 | C20625731 | SMD3225-4P | 1 | Controller crystal. Match load capacitors to selected crystal and RP2350 reference design. |
| USB ESD protection | USBLC6-2SC6 | C7519 | SOT-23-6L | 1 | USB 2.0 D+/D- ESD protection near USB-C connector. |
| USB-C receptacle | TYPE-C-31-M-12 | C165948 | USB-C 16-pin | 1 | USB 2.0 device connection to RP2350. Verify footprint and CC resistor wiring. |
| GPIO buffer | SN74LVC244APWR | C7668 | TSSOP-20 | 2 | Buffers all RP2350-controlled enable outputs. 16 total buffer channels. |
| E-stop AND gate | SN74LVC08APWR | C465737 | TSSOP-14 | 3 | Gates final enables with E_STOP_OK. 12 total AND channels. |
| High-side gate driver | ADI LTC7001IMSE#PBF Check LIPO MAX vs MIN to make sure the cutoff is in range. | C581062 | MSOP-10-EP | 8 | One per drivetrain branch, ARM rail, and SCIENCE rail. Verify supply rail and gate drive. |
| Power MOSFET | IPT007N06N Since this mosfet after about ~8v reaches its “peak” current capabilities, you can set the vcc of this to be on the 12v rail output | C122552 | HSOF-8 | 8 | One per drivetrain branch, ARM rail, and SCIENCE rail. |
| I2C power monitor | INA780BIDEKR | C34740774 | VQFN-15 6x6 | 8 | One per drivetrain branch, ARM rail, and SCIENCE rail. |
| I2C buffer | TCA4307DGKR | C880333 | VSSOP-8 | 2 min. | One per RP2350 I2C bus segment. Add more only if layout requires more physical segmentation. |
| Temperature sensor | MCP9808T-E/MS | C129490 | MSOP-8 | 3 min. | Logic, drivetrain, and arm/science thermal zones. |
| Buck regulator | LT8612EUDE#TRPBF | C462617 | QFN-28-EP 3x6 | 3 | 20v for Jetson, 3.3v for rp2350, and 12v for the POE switch + network switch. Jetson power + networkswitch + POE will use screw terminal blocks |
| XT60 connector | XT60PW-M | C98732 | Through-hole power | 11 typical | Three battery inputs, six drivetrain outputs, ARM output, SCIENCE output. Final orientation: vertical/up-facing where specified. |
| E-stop terminal | KF301-5.0-2P | C474881 | 2-pin screw terminal | 1 | Two-pin terminal for pull-to-GND E-stop switch. |
| Screw Terminal Block | KF301-5.0-2P | C474881 | 2-pin screw terminal | 3 | Jetson power, POE Switch, network switch |

## 4.1 Removed / Rejected Parts

| Part | Decision |
| --- | --- |
| MCP23017 GPIO expander | Removed from baseline. Native RP2350 GPIO is sufficient. |
| TPS1HB08 for drivetrain or arm/science switching | Removed from high-current paths. IPT007N06N + gate driver stack is baseline. |
| Allegro ACS37800 / ACS71020 | Removed. ACS37800 path was deprecated/not preferred for this design; ACS71020 is not a new-design replacement. |
| Direct GPIO-to-MOSFET gate drive | Rejected. High-side N-MOSFETs require a proper gate driver. |

# 5. Sensor Specifications and I2C Address Map

## 5.1 I2C Topology Requirement

The design shall use onboard TCA4307 I2C buffers.

The RP2350 shall use both I2C controllers in the baseline design.

TCA4307 devices are buffers, not multiplexers. They do not solve duplicate I2C addresses.

All devices sharing a logical I2C bus shall have unique addresses.

High-current telemetry shall be physically separated from local controller and temperature sensors when practical.

No GPIO expander shall be placed on the required baseline I2C bus.

Recommended I2C allocation:

RP2350 I2C0 - high-current telemetry bus
   `-- TCA4307 buffer: drivetrain + arm/science segment

|-- INA780x LOGIC
       |-- INA780x DRIVE1
       |-- INA780x DRIVE2
       |-- INA780x DRIVE3
       |-- INA780x DRIVE4
       |-- INA780x DRIVE5
       |-- INA780x DRIVE6
       |-- INA780x ARM
       `-- INA780x SCIENCE

RP2350 I2C1 - local/logic telemetry bus
   `-- TCA4307 buffer: logic/temp segment
       |-- MCP9808 logic zone
       |-- MCP9808 drivetrain zone
       `-- MCP9808 arm/science zone

## 5.2 Target I2C Address Map

| Bus | Device | 7-bit address | Address strap |
| --- | --- | --- | --- |
| I2C0 | INA780x DRIVE1 | 0x40 | A1=GND, A0=GND |
| I2C0 | INA780x DRIVE2 | 0x41 | A1=GND, A0=VS |
| I2C0 | INA780x DRIVE3 | 0x43 | A1=GND, A0=SCL |
| I2C0 | INA780x DRIVE4 | 0x44 | A1=VS, A0=GND |
| I2C0 | INA780x DRIVE5 | 0x45 | A1=VS, A0=VS |
| I2C0 | INA780x DRIVE6 | 0x47 | A1=VS, A0=SCL |
| I2C0 | INA780x ARM | 0x4C | A1=SCL, A0=GND |
| I2C0 | INA780x SCIENCE | 0x4D | A1=SCL, A0=VS |
| I2C0 | INA780x LOGIC | 0x4F | A1=SCL, A0=SCL |
| I2C1 | MCP9808 LOGIC_TEMP | 0x18 | A2=0, A1=0, A0=0 |
| I2C1 | MCP9808 DRIVE_TEMP | 0x19 | A2=0, A1=0, A0=1 |
| I2C1 | MCP9808 ARM_SCI_TEMP | 0x1A | A2=0, A1=1, A0=0 |

INA780x addresses using SDA straps shall be avoided unless explicitly required, because TI notes an additional hold-time requirement when SDA is used as an address strap.

The firmware shall probe all expected addresses at boot and fail safe if any required monitor is missing.

## 5.3 Required Telemetry Values

Per drivetrain branch: voltage, current, power, energy/charge if enabled, alert/fault status.

Computed drivetrain total: sum of six branch currents and sum of six branch powers.

ARM rail: voltage, current, power, energy/charge if enabled, alert/fault status.

SCIENCE rail: voltage, current, power, energy/charge if enabled, alert/fault status.

Temperature: logic regulator zone, drivetrain MOSFET/INA zone, arm/science MOSFET/INA zone.

E-stop status: asserted/not asserted.

GPIO status: commanded state and final allowed state for each controlled output.

# 6. RP2350 GPIO Allocation and Software Bindings

## 6.1 GPIO Capacity Requirement

The baseline design shall use native RP2350 GPIO only.

The RP2350A QFN-60 package exposes enough GPIO for all required enable, I2C, E-stop, debug, and status signals.

GPIO expanders shall not be used for required enable control.

If a future revision adds enough fault inputs to exceed native GPIO capacity, add a GPIO expander only for noncritical status inputs, not for enable lines.

## 6.2 Proposed Native GPIO Map

| RP2350 pin | Net name | Function |
| --- | --- | --- |
| GPIO0 | I2C0_SDA | High-current telemetry bus SDA |
| GPIO1 | I2C0_SCL | High-current telemetry bus SCL |
| GPIO2 | I2C1_SDA | Logic/temp telemetry bus SDA |
| GPIO3 | I2C1_SCL | Logic/temp telemetry bus SCL |
| GPIO4 | DRIVE_EN_1 | Native output -> buffer -> E-stop gate -> gate driver |
| GPIO5 | DRIVE_EN_2 | Native output -> buffer -> E-stop gate -> gate driver |
| GPIO6 | DRIVE_EN_3 | Native output -> buffer -> E-stop gate -> gate driver |
| GPIO7 | DRIVE_EN_4 | Native output -> buffer -> E-stop gate -> gate driver |
| GPIO8 | DRIVE_EN_5 | Native output -> buffer -> E-stop gate -> gate driver |
| GPIO9 | DRIVE_EN_6 | Native output -> buffer -> E-stop gate -> gate driver |
| GPIO10 | ARM_EN | Native output -> buffer -> E-stop gate -> gate driver |
| GPIO11 | SCIENCE_EN | Native output -> buffer -> E-stop gate -> gate driver |
| GPIO12 | AUX_22_EN | Native output -> buffer -> E-stop gate -> aux switch/driver |
| GPIO13 | SPARE_EN (NOT NEEDED) | Buffered spare output; route to test pad or spare gate channel |
| GPIO14 | E_STOP_STATUS | Input from E_STOP_NODE / E_STOP_OK conditioning |
| GPIO15 | PDB_FAULT_OUT | Output to Jetson or external fault indicator |
| GPIO16 | JETSON_HEARTBEAT_IN | Optional input from Jetson |
| GPIO17 | TCA4307_I2C0_EN | Optional enable/reset for I2C0 buffer |
| GPIO18 | TCA4307_I2C1_EN | Optional enable/reset for I2C1 buffer |
| GPIO19 | STATUS_LED | Controller heartbeat LED |
| GPIO20 | FAULT_LED | Fault/status LED |
| GPIO21 | DEBUG_UART_TX | Optional debug UART |
| GPIO22 | DEBUG_UART_RX | Optional debug UART |
| GPIO23 | DRIVE_FAULT_SUM_IN | Optional wired/fault summary input |
| GPIO24 | ARM_SCI_FAULT_SUM_IN | Optional wired/fault summary input |
| GPIO25 | LOGIC_FAULT_IN | Optional logic-domain fault input |
| GPIO26 | VLOGIC_ADC_SPARE | Optional ADC sanity check / spare |
| GPIO27 | VDRIVE_ADC_SPARE | Optional ADC sanity check / spare |
| GPIO28 | VARM_SCI_ADC_SPARE | Optional ADC sanity check / spare |
| GPIO29 | SPARE_ADC_OR_BUTTON | Spare ADC or user/service button |

## 6.3 Buffer and AND Gate Channel Use

| Signal | SN74LVC244 channel | SN74LVC08 channel | Final destination |
| --- | --- | --- | --- |
| DRIVE_EN_1 | BUF1-A1 | AND1-1 | Gate driver DRIVE1 input |
| DRIVE_EN_2 | BUF1-A2 | AND1-2 | Gate driver DRIVE2 input |
| DRIVE_EN_3 | BUF1-A3 | AND1-3 | Gate driver DRIVE3 input |
| DRIVE_EN_4 | BUF1-A4 | AND1-4 | Gate driver DRIVE4 input |
| DRIVE_EN_5 | BUF1-B1 | AND2-1 | Gate driver DRIVE5 input |
| DRIVE_EN_6 | BUF1-B2 | AND2-2 | Gate driver DRIVE6 input |
| ARM_EN | BUF1-B3 | AND2-3 | Gate driver ARM input |
| SCIENCE_EN | BUF1-B4 | AND2-4 | Gate driver SCIENCE input |
| AUX_22_EN | BUF2-A1 | AND3-1 | Aux switch/driver input |
| SPARE_EN | BUF2-A2 | AND3-2 | Spare gated output |

Tie unused SN74LVC244 inputs to a defined state.

Disable unused SN74LVC244 outputs or route them to labeled test pads.

Tie unused SN74LVC08 inputs to GND. Do not leave CMOS inputs floating.

All final enable lines shall have local pulldowns so power stages default OFF when logic is unpowered.

## 6.4 Zephyr / F Prime Binding Names

Required firmware signal names:

i2c_drive = I2C0
  ina780_drive_1 @ 0x40
  ina780_drive_2 @ 0x41
  ina780_drive_3 @ 0x43
  ina780_drive_4 @ 0x44
  ina780_drive_5 @ 0x45
  ina780_drive_6 @ 0x47
  ina780_arm     @ 0x4C
  ina780_science @ 0x4D

i2c_local = I2C1
  temp_logic     @ 0x18
  temp_drive     @ 0x19
  temp_arm_sci   @ 0x1A

gpio outputs:
  drive_en_1..drive_en_6
  arm_en
  science_en
  aux_22_en
  pdb_fault_out
  tca4307_i2c0_en
  tca4307_i2c1_en

gpio inputs:
  e_stop_status
  jetson_heartbeat_in
  drive_fault_sum_in
  arm_sci_fault_sum_in
  logic_fault_in

# 7. Connection Requirements and PCB Guidelines

## 7.1 Manufacturing Baseline

| Parameter | Requirement |
| --- | --- |
| Layer count | 4-layer minimum. 6-layer preferred if budget allows. |
| Copper weight | 2 oz outer layers minimum for high-current sections. Use heavier copper or reinforcement if thermal/current review requires it. |
| Board size | Any size is acceptable if the board fits within at most one-third of the rover footprint. |
| High-current connectors | Use vertical through-hole XT60 for drivetrain outputs and high-current ARM/SCIENCE outputs unless explicitly approved otherwise. |
| Logic outputs | Use screw terminal blocks or robust terminal connectors. Do not use small logic terminals for motor power. |
| Assembly | Do not use thermal reliefs on high-current MOSFET, INA780x high-current, or XT60 power pads unless assembly review requires it. |

## 7.2 Recommended Stackup

4-layer baseline:
L1: components, local high-current pours, switching/gate-driver components
L2: solid GND plane
L3: power planes / high-current distribution zones / 3.3 V where needed
L4: signals, low-current routing, secondary pours

6-layer preferred:
L1: components + local high-current routing
L2: solid GND
L3: high-current power planes / 22.2 V / 14.4 V zones
L4: 3.3 V / quiet logic / sensor routing
L5: solid GND
L6: signals + secondary power pours

## 7.3 Routing Rules

| Net class | Starting rule |
| --- | --- |
| USB D+/D- | Route as controlled-impedance differential pair. Place USB ESD near connector. Keep stubs short. |
| QSPI flash | Short routes, length matched only as needed by reference design. Keep away from high-current switching copper. |
| Crystal | Place next to RP2350. Keep loop tiny. Guard with ground. Follow reference design load-cap values. |
| GPIO enables | 6-8 mil minimum. Route from RP2350 to buffer, then to AND gate, then to gate driver input. |
| I2C | 6-8 mil minimum. Pullups on buffered side as required. Keep high-current I2C physically separated from logic bus. |
| Gate drive | Short, direct, local to MOSFET. Use 5-10 ohm starting gate resistor. Add gate-source pulldown. |
| INA780x current path | Treat as power copper, not signal. Follow TI layout. Use short, wide pours and thermal copper. |
| Motor power | Wide pours or busbar-like copper. Use 2 oz copper minimum. No thin traces. |

## 7.4 Current Path Rules

Calculate I^2R loss for every IPT007N06N branch using worst-case branch current.

Calculate heat in the INA780x package and copper for every monitored branch.

Place MOSFET, gate driver, INA780x, and XT60 for each branch in a physically compact power cell.

Keep current loops short and wide.

Do not route USB, QSPI, crystal, or I2C under or beside motor-current hot paths unless separated by solid ground and distance.

Provide test points for every enable, final gated enable, gate-driver input, E_STOP_NODE, I2C bus, and all major rails.

# 8. KiCad Schematic Sheet Plan

| Sheet | Required contents |
| --- | --- |
| 00_BILLEE_ROVER_MODULE_V1 | Top-level hierarchy, connectors between sheets, global net naming, revision block. |
| 01_INPUTS_ESTOP | Three XT60 battery inputs, E-stop screw terminal, E_STOP_NODE conditioning, full-system disconnect interface, input TVS/protection. |
| 02_RP2350_CONTROLLER | RP2350A, QSPI flash, 12 MHz crystal, USB-C, USB ESD, BOOTSEL/RUN, SWD header, 3.3 V decoupling, reference-design compliance notes. |
| 03_GPIO_BUFFER_ESTOP_GATE | SN74LVC244 buffers, SN74LVC08 AND gates, pulldowns, test points, final enable naming. |
| 04_I2C_BUFFERS | I2C0/I2C1 pullups, TCA4307 buffers, READY/EN pins, bus test points. |
| 05_LOGIC_22V | 22.2 V logic rail, Jetson/switch/POE regulator paths, 12 V 10 W POE path, aux output. |
| 06_DRIVE_BRANCH_1_2 | Gate drivers, IPT007N06N, INA780x, XT60 outputs for drive branches 1 and 2. |
| 07_DRIVE_BRANCH_3_4 | Gate drivers, IPT007N06N, INA780x, XT60 outputs for drive branches 3 and 4. |
| 08_DRIVE_BRANCH_5_6 | Gate drivers, IPT007N06N, INA780x, XT60 outputs for drive branches 5 and 6. |
| 09_ARM_SCIENCE | ARM gate driver + IPT007 + INA780x + output; SCIENCE gate driver + IPT007 + INA780x + output. |
| 10_TEMPERATURE_TELEMETRY | MCP9808 sensors, address straps, placement notes, I2C1 connection. |
| 11_MECHANICAL_TEST | Mounting holes, board outline, fiducials, test pads, labels, connector orientation notes. |

# 9. Software Implications

## 9.1 Firmware Baseline

Firmware shall support Zephyr on RP2350A or a custom board target derived from Raspberry Pi Pico 2 / RP2350A support.

F Prime shall run above the board support layer if used. F Prime components shall not bypass safety state management.

All enable GPIO shall default OFF at reset.

Firmware shall read E_STOP_STATUS before enabling any controlled rail.

Firmware shall not attempt to override E-stop gating.

## 9.2 Required Firmware Modules

| Module | Required behavior |
| --- | --- |
| Boot manager | Initialize clocks, GPIO directions, I2C buses, TCA4307 enables, telemetry devices, and default safe states. |
| Rail controller | Own commanded states for DRIVE1-6, ARM, SCIENCE, AUX_22. Enforce sequencing and E-stop rules. |
| INA780x driver | Read voltage, current, power, energy/charge if enabled, temperature/alert registers as required. |
| MCP9808 driver | Read logic, drive, and arm/science temperatures. |
| Telemetry service | Publish per-rail and computed drivetrain totals to Jetson over USB serial. |
| Command parser | Accept structured commands from Jetson. Reject commands when E-stop is asserted or telemetry is invalid. |
| Fault manager | Latch overcurrent, overtemperature, missing sensor, I2C failure, E-stop, and heartbeat faults. |
| Watchdog | Reset or force safe state on firmware lockup. |

## 9.3 Required Jetson Command Set

Minimum commands:

status
telemetry
faults
faults clear
rail drive1 on|off
rail drive2 on|off
rail drive3 on|off
rail drive4 on|off
rail drive5 on|off
rail drive6 on|off
rail drive_all on|off
rail arm on|off
rail science on|off
rail aux22 on|off
high_power disable
heartbeat
reboot_controller

# 10. Validation and Bring-Up Plan

Bring up RP2350 controller alone from current-limited bench supply.

Verify USB-C enumeration and bootloader access.

Verify SWD programming and debug access.

Run GPIO self-test into buffers and AND gates with no power stages populated if possible.

Verify E_STOP_NODE high when released and low when pressed.

Verify every final enable is forced low when E-stop is pressed.

Bring up I2C0 with no high-current load. Probe all INA780x addresses.

Bring up I2C1 and probe all MCP9808 addresses.

Populate one drivetrain branch and test gate-driver/MOSFET switching with current-limited load.

Validate INA780x readings against bench supply/current meter.

Repeat for all drivetrain branches.

Validate ARM and SCIENCE rails separately.

Run thermal test at expected continuous current.

Run transient/inrush test with motor controller attached.

Only after branch validation, connect real motors and verify software sequencing.

# 11. Design Checklist / Assignment Rubric

## 11.1 Procurement / Library Checklist

Every required part exists in LCSC/JLCPCB or has mentor-approved alternate sourcing.

Every LCSC part number in Section 4 has been rechecked at design freeze.

Every symbol exists in KiCad and matches the datasheet pinout.

Every footprint exists in KiCad and matches the manufacturer land pattern.

Every high-current connector footprint has been checked against the physical part.

RP2350A footprint, exposed pad, USB, flash, crystal, and power pins have been checked against the Raspberry Pi reference design.

## 11.2 Schematic Checklist

RP2350A support circuit follows official/reference schematic.

USB-C CC resistors, ESD, D+/D- routing, and power behavior are correct.

QSPI flash and crystal are correct and placed on the controller sheet.

No MCP23017 or GPIO expander exists in the required control path.

All enable outputs pass through SN74LVC244 before SN74LVC08 gates.

All E-stop gated enables have pulldowns and safe default states.

Every INA780x has unique address straps matching Section 5.

Every TCA4307 has correct pullups, EN/READY handling, and test points.

Every gate driver has verified logic input and verified power supply connections.

Every MOSFET has gate resistor, gate-source pulldown, and optional gate-source clamp if required.

## 11.3 PCB Checklist

High-current branch cells are physically compact and repeated consistently.

MOSFETs, INA780x packages, and XT60 connectors use wide copper and thermal copper.

USB, QSPI, crystal, and I2C are routed away from high-current paths.

Ground plane is continuous under RP2350, USB, QSPI, crystal, and I2C.

No thermal reliefs on critical power copper unless approved.

All connectors have readable silkscreen labels and polarity markings.

Test points exist for all rails, E-stop, I2C, enable signals, final enables, and fault summaries.

Board fits within at most one-third of rover footprint.

## 11.4 Software Checklist

Zephyr board definition or F Prime deployment matches the GPIO/I2C names in Section 6.

All rails default OFF except fixed always-on paths when not E-stopped.

Firmware refuses to enable outputs when E-stop is asserted.

Firmware probes all I2C devices and reports missing sensors.

Firmware computes drivetrain total current and power from branch telemetry.

Firmware implements high_power disable command.

Telemetry packet includes rail states, voltage, current, power, temperature, fault state, and E-stop state.

# 12. Open Verification Items

| Open item | Action required |
| --- | --- |
| Full-system E-stop disconnect | The screw-terminal pull-to-GND input is defined. The contact is pulled to ground but the AND gate is wired to a pull up, so when the E-STOP switch closes, all the EN lines go down. |
| Gate-driver supply | The selected gate driver logic input is controlled by 3.3 V. Its power supply must still be verified so the IPT007N06N is fully enhanced. |
| IPT007 thermal margin | Calculate I^2R loss for expected branch current and verify copper/thermal design. Do not trust package headline current rating alone. |
| INA780x thermal margin | Verify each branch current profile is inside INA780x measurement and thermal envelope. |
| RP2350 reference design merge | Compare final controller sheet against Raspberry Pi official minimal design and the Bronco Space Lab BLVLink hardware v2.0 reference design before PCB routing. |
| Zephyr/F Prime build target | Create custom RP2350A Zephyr board or derive from rpi_pico2/rp2350a. Confirm USB CDC and both I2C buses before vehicle integration. |

# 13. References

| Reference | Used for |
| --- | --- |
| Raspberry Pi RP2350 product brief | Confirms RP2350 package variants, 30/48 GPIO variants, dual Arm/RISC-V architecture, PIO count. |
| Raspberry Pi Hardware Design with RP2350 | Controller reference design for RP2350A/RP2350B minimal boards, flash, crystal, USB, SWD, BOOTSEL/RUN. |
| Bronco Space Lab BLVLink hardware v2.0 GitHub repository | RP2350 reference design cross-check. Use this repository to compare the custom controller sheet against known RP2350 support circuitry, including MCU schematic structure, USB, flash, crystal, SWD, BOOTSEL/RUN, and KiCad project organization before PCB routing. |
| Zephyr Raspberry Pi Pico 2 documentation | Closest Zephyr RP2350A board target and feature reference. |
| TI INA780B datasheet | I2C power monitor specs, address table, current/voltage/power telemetry, layout requirements. |
| Infineon IPT007N06N datasheet / LCSC listing | MOSFET voltage, package, RDS(on), current class, gate charge. |
| ADI LTC7001 datasheet / LCSC listing | High-side N-MOSFET driver behavior and supply requirements. |
| TI SN74LVC244A and SN74LVC08A datasheets / LCSC listings | 3.3 V GPIO buffer and E-stop AND gate logic. |
| TI TCA4307 datasheet / LCSC listing | I2C buffer, bus recovery, hot-swap/buffering behavior. |
| Microchip MCP9808 datasheet / LCSC listing | I2C temperature sensor behavior and address options. |
| Abracon ABM8-272-T3 listing | 12 MHz crystal selected for RP2350 reference-design-compatible clock circuit; LCSC C20625731. |